CNRS PPRIME: BACKWARD FACING STEP

The model of CNRS PPRIME has a step height of 30 mm and a span wise of 300 mm. The experimental facility used is a closed-loop wind tunnel with a test section of 300x300x1000 mm3.The turbulence intensity is less than 1%. The aspect and expansion ratio are 10 and 1.11 respectively. The figure 1 shows the model installed in the wind tunnel and the principal parameters.

Figure 1 Step model in the test section.

The experiments were carried out for a mean free-stream velocity of 15 m/s and it is considered as the reference velocity (Uref). The Reynolds number based on this velocity and the step height (h=30 mm) is equal to 3x104. The mean reattachment point occurs at x/h=5.85 as seen in figure 2 by streamlines and it is modified up to 4.68 for the case of burst modulation, representing a reduction of 20%.

Figure 2 Streamlines, U=0 m/s line and reattachment point for uncontrolled and controlled case.

The Reynolds stress component, <uv>, and the turbulent kinetic energy, k, for uncontrolled and controlled case are represented in figure 3 and 4 respectively.

Figure 3 Reynolds stress spatial distribution for the component <uv>.

Figure 4 Turbulent kinetic energy distribution.