NACA0015 + Synthetic Jet TestCase Definition

Case coordinators: Prof. Tang Zhili, NUAA

Prof. Liu Guozheng, ARI

The included test case was conducted by NUAA and ARI.

# Computational domain

- upstream: 20*C far away from leading edge,

- downstream: 20*C far away from trailing edge,

- top boundary: 20*C far away from chord line,

- bottom boundary: 20*C far away from chord line.

- C is the chord length, C=500mm

- Spanwise length =200mm

Figure 1. Diagram of computational domain

# Boundary condition

- left and right boundary: periodic boundary condition

- upstream, down stream, top and bottom boundary: far field boundary condition, free stream velocity=10,20,30m/s, angle of attack=18deg.

- wall: no-slip boundary condition

# Flow analyzer

- 3D URANS with Turbulence Model

# Devices geometry of SJ

- Defined by experiment

- Width of slot: 1.5mm

# Design variables

- frequency of jet: f,

- amplitude of jet flow: V_Jet,

- direction of jet flow: Theta,

- position of jet flow exit: p.

Figure 2. Control device parameters for optimization

# Search space

- V_jet=[30m/s,80m/s],

- frequency=[30Hz,60Hz],

- Theta=[30deg,160deg],

- Position of jet: p=[5%C-7%C]

# Objective function: J

- Maximize J=averaged lift:

# Constraints

- No constraints

# Optimizer

- Genetic algorithms with adaptive surrogate model

# Data outputs (to be decided)

- Cl with respect to t (time)

- Cd with respect to t (time)

- several periods of zoomed flow fields

- separation point

- separation region

- …….

# Outputs data formats

- Tecplot data formats (to be decided)

Remark: 2D NACA0015 airfoil + SJ is also available from the above definition with the same parameters of the same device design parameter of fig. 2.